Veecom E1/T1 over 2 Ethernet adaptive

V-IPMUX-SA01/02 adopt the innovative TDM over IP technology, with IP circuit emulation that supports transportation of 1~2 E1 /T1 over FE port, that can be used to provide E1 communication channels over Ethernet or IP networks.

Features:

- IPMUX-SA01 provide 1 channel of E1/T1 over 1 Ethernet adaptive

- IPMUX-SA02 provide 2 channels of E1/T1 over 1 Ethernet adaptive

- Provide 4 FE electrical ports (2 for uplink, 2 for user data or monitoring) and 1 Ethernet optical port optical port can be used for uplink or user data.

- Support IETF RFC4553 SAToP protocol, Ethernet encapsulation support IP/UDP

- User-friendly Web server supported for easy setup and maintenance

- Support SNMP V1/V2 network management

- E1 clock supports 3 mode: local clock, adaptive and loopback

- E1 service support lossless protection

- Ethernet built-in layer 2 switch, support VLAN(802.1Q based and QinQ based), QoS(port based、8021.1P based and TOS based)

- Ethernet support IEEE 802.3x and MAC address automatic learning

- Ethernet packet size up to 9720byte

- Point to point and point to multipoint supported

- Software and hardware online upgrade

- Support SNTP network time setting

- Support software test E1 error

- Support VLAN settings for E1 service and in band VLAN management

- Enough jitter buffer to resist packet delay variation (PDV)

- Local Ethernet port throughput limiting, assuring E1 QoS

- 120ohm balanced E1/T1 port, RJ-45 connector, support 75ohm unbalanced port through outside converting cable

- Support cascade concatenate for more than 16 E1 ports

- Software and hardware online upgrade

- Power supply redundancy.

Typical Application

Technical Specifications

Model

- V-IPMUX-SA01/02: 2 uplink, 1/2 E1/T1s

Service card

- Ethernet interfaces

- Speed and duplex auto-netiation or manual

- Comply with IEEE 802.3, 802.1Q, 802.1P, 802.3x

- 4 FE electrical ports and 1 FE optical port

- E1 interface

- V-IPMUX-SA01: 1 E1 ports

- V-IPMUX-SA02: 2 E1 ports

Power

- Supply

- AD or DC

- DC  -48V (-32V~-72V)

- AC  220V (100V~265V)

- Consumption: ≤40W

Working Environment

- Temperature: 0~ 50°C

- Relative Humidity: ≤90% (non-condensing)

Dimension

- W x H x D (mm):  185 x 35 x 136.5 (mm)